Environmental Hazard Detection — Dataset & Model Reference

Project: Seraph Zenith Mark 1 — Environmental Hazards Module Target hardware: Raspberry Pi 5 + AI HAT+ (Hailo-8) Approach: Single unified YOLOv8 multi-class detection model

Base model. The unified model is built on YOLOv8n (nano), from Ultralytics (https://github.com/ultralytics/ultralytics). It's lightweight enough to leave compute headroom for the ORB matching, LoFTR cueing, and EKF fusion running concurrently on the same Pi 5, and it's the exact architecture Hailo Model Zoo's retraining recipe expects with no structural changes required.

Fire / Smoke. The base dataset is D-Fire (https://github.com/sayedgamal99/DFireDataset), which gives 21,000+ YOLO-labeled images covering a wide range of fire and smoke appearances across day/night and weather conditions. Because D-Fire is mostly ground-level and surveillance-camera footage rather than aerial, it's supplemented with UAVs-FFDB (https://openaccess.hku.edu.tr/items/30214f7b-6b8e-4a4d-b221-bb9aedf17547), a genuinely drone-captured forest fire dataset shot from 5–15m altitude, to close the viewpoint gap between ground-level training data and your actual survey camera angle. As a reference baseline, Pyronear's YOLOv8s (https://huggingface.co/pyronear/yolov8s) is already fine-tuned on D-Fire and useful for a quick sanity check before your own training run.

Flood. The dataset is Drone-for-flood on Roboflow (https://universe.roboflow.com/drone-7nzsq/drone-for-flood), chosen because it's explicitly drone-captured and matches your actual survey altitude and viewpoint rather than requiring a domain-transfer assumption. No public flood-specific pretrained weights exist, so this class is fine-tuned directly from the yolov8n.pt COCO checkpoint.

Damaged Structures. The dataset is the UAV-Centre-ITC Damaged Building Opening dataset (https://github.com/UAV-Centre-ITC/yolo-damaged-building-opening-detection), built specifically for UAV-based urban search and rescue — the same operational context as your drone. The same repository ships ready-to-use YOLOv5-MTL and YOLOv5-single pretrained weights, which is valuable both as a working baseline and because YOLOv5 is natively supported in Hailo Model Zoo's conversion pipeline.

Debris. The dataset is RescueNet (https://www.nature.com/articles/s41597-023-02799-4), a UAV-collected, high-resolution post-disaster dataset that includes debris as an explicit pixel-level class rather than an incidental one. No pretrained debris-specific model exists, so RescueNet's segmentation masks are converted to bounding boxes (via cv2.findContours + boundingRect) and folded into the unified YOLOv8n model rather than training a separate segmentation head, keeping the on-device pipeline to a single detection model.

Exposed Electrical Lines. The dataset is MPID, the Merged Public Insulator Dataset (https://github.com/phd-benel/MPID), built from real UAV inspection imagery of transmission-line hazards and already provided in YOLO-compatible bounding-box format. No pretrained weights are publicly available for this class, so it's fine-tuned directly from yolov8n.pt; the label format is a direct match for your training pipeline with no reformatting needed.

Landslide. The dataset is the CAS Landslide Dataset (https://www.ncbi.nlm.nih.gov/pmc/articles/PMC10762236/), the largest available UAV-and-satellite landslide collection at 20,865 images, chosen to offset the fact that landslides are your rarest real-world event class and need the extra volume to train reliably. No pretrained model exists, so segmentation masks are converted to bounding boxes and fine-tuned from yolov8n.pt.

Chemical Spill / Leak. The dataset is chem-spill on Roboflow (https://universe.roboflow.com/k-d113o/chem-spill), the only free RGB (non-thermal) visible-spill dataset available — most real chemical and gas leak detection research relies on thermal/IR imaging, so this is the closest visible-light proxy your RGB-only setup can use. At only 56 images it's by far the smallest dataset in the set, so this class should be treated as your lowest-confidence one and will need heavy augmentation to avoid overfitting.

Auxiliary references (not used for training). AIDER / AIDERv2 (https://github.com/ckyrkou/AIDER) is a whole-image disaster classification dataset (fire/flood/earthquake/normal) — useful as a coarse cross-check on your model's outputs, but not usable for bounding-box training since it has no localization labels. LADI v2 (https://arxiv.org/pdf/2406.02780) is low-altitude oblique aerial imagery with a debris_any multi-label tag and comes with existing pretrained classifiers, making it a reasonable secondary confidence signal, though it likewise can't localize a detection the way your pipeline needs.

Hailo-8 conversion path. Regardless of class, the export sequence is the same: convert the trained best.pt to ONNX with yolo export model=best.pt format=onnx opset=11, parse it with hailomz parse yolov8n --ckpt best.onnx --hw-arch hailo8 --start-node-names images --end-node-names <output nodes>, then compile to HEF with hailomz compile --ckpt best.onnx --calib-path <calibration_images> --yaml yolov8n.yaml. Set model_optimization_flavor(optimization_level=1, compression_level=0, batch_size=2) in the .alls script to avoid the known accuracy drop that low-class-count YOLOv8 models see after quantization, and make sure the calibration image set is sampled proportionally across all six classes rather than letting the largest dataset (D-Fire) dominate it. Reference repo: https://github.com/hailo-ai/hailo_model_zoo.